Hospital Indoor Localization & Behavior Analysis

USE CASE – V2.

Medical Environments

Patient-Related

- Activity Tracking

- Movement between room → corridor → public areas

- Analysis of speed, activity frequency, and dwell points

- Risk Event Detection

  - Fall detection

  - Alerts when patients enter restricted zones (ICU or isolation rooms…)

Staff

  - Rounding path monitoring / identifying

  - Workload & crowded areas detection

  - Process optimization

Medical Equipment

- Real-Time Asset Tracking

  - Wheelchairs

  - beds…

- Usage Pattern Analysis

  - Usage frequency

  - Movement cycles

Constraints in Hospital Environments

Radio Environment Instability

- Surveyed literature emphasizes that indoor environments (especially hospitals) cause high variability in wireless signals due to obstacles and dynamic conditions.

Source: A Survey on Indoor Positioning Systems for IoT-Based Applications

- Frequent movement of beds, metallic equipment, human crowds, and door opening/closing causes multipath, fading, and severe RSSI fluctuations.

Interference from Medical Devices

- Imaging devices and electronic medical equipment generate electromagnetic emissions that may interfere with localization technologies.

Privacy and Ethical Limitations

- Sensors must comply with medical privacy standards.

- Wearable-based solutions for patients may require consent and approval.

Deployment & Operational Constraints

- Hospitals often avoid major infrastructure changes.

- low-maintenance and low-cost solutions.

- Multiple technologies (Wi-Fi, BLE, IR, ultrasound) coexist (increasing system management complexity).

Existing Localization Solutions (in Hospitals)

| Wi-Fi RSSI-Based Localization | - RSSI value. - Simple, widely available due to existing Access Points. - Typical accuracy: 2–4 meters in real indoor conditions. - High interference, limited accuracy. |
| --- | --- |
| RSSI Fingerprinting Systems | - Offline fingerprint database + online matching. - Achieves sub-meter accuracy (≈ 60 cm under ideal conditions). - Highly sensitive to environmental changes. - Labor-intensive to maintain. |
| Bluetooth Low Energy (BLE) | - RSSI beacon sequences (requires beacon deployment) - Low deployment cost, low power consumption. - Works well for room-level or corridor-level localization - Sensitive to environmental dynamics. |
| Ultra-Wideband (UWB) | - Uses ToF / TDoA measurements - Provides high accuracy (cm-level). - High cost & infrastructure requirements |
| IMU-Based Behavioral Monitoring | - IMU accelerometers/gyroscopes → fall detection - Requires wearable devices |

Sensors and Methods Used in Hospital Localization

Sensors

Methods

- Distance & Angle-Based Approaches

- ToA / TDoA: Used in UWB; requires synchronization.

- AoA: Supported by BLE 5.1 antenna arrays; higher deployment cost.

- RSSI Fingerprinting

- Widely used in hospitals: easy to deploy

- Common algorithms include KNN, SVM, probabilistic models, and deep learning.

- CSI-Based Localization

- Provides richer channel information

- Improving stability and accuracy.

- Tracking & Filtering Methods

- Kalman Filter, Particle Filter for smoothing trajectories.

- Widely used in real-time tracking systems: noise robustness.

- Multi-Sensor Fusion

- Combining Wi-Fi + BLE + IMU or UWB ….

- Improves robustness in dynamic hospital environments.

- Multipath, occlusions, and varying conditions…